# tlog2chart Phase 3 Basic Analysis Report

File: 01-Tykon0527-004-2_53300201.txt
Unit Type: Tykon1213
Unit FW Ver: 34
Unit FPGA Ver: 5
Unit SN: 53300201
Tool: tlog2chart_P3 v1.3.6
Generated: 2026-03-19 23:15:31

# Analysis Settings

- Active threshold: Pfwd > 3.00 W
- Step4 tolerance: ±2.0%  | settle time: 6.0 ms (Tykon only)
- Step5 reflect rule: Pref > 1.00% of Pfwd  | mask first 3.0 ms
- Pulse mode range: Freq 0.01–50.00 kHz, Duty 10.0–90.0%

# Step 1 — Full Scale Overview

Full plot of Pfwd/Pref/Pdel across the entire tlog.

Figure 1. Full-scale power overview.

# Step 2 — Zoom In to last RF event with 100x smaller time span than Step 1, improving resolution around the last power event.

This plot zooms into the time window around the last RF event. The X-axis time span is 100× smaller than Step 1 to improve resolution near the last power activity.

Figure 2. Active-area power plot (idle removed).

# Step 3 — First 6 RF Cycles

Step 3 cycle metrics (base on RF on/off signal):

| Cycle | Start row() | SetPt (W) | Pfwd steady (W) | Pfwd peak (W) | Overshoot (W) | Overshoot (%) | Rise to 95% (ms) | RF ON (ms) | Idle before (ms) | Mode / Freq / Duty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15.0 | 14.1 | 24.7 | 9.7 | 64.67 | 0.750 | 50.000 | N/A | Pulse, F=0.400kHz, D=15.0% |
| 2 | 210 | 200.0 | 200.0 | 202.4 | 2.4 | 1.20 | 2.250 | 150.000 | 150.000 | CW |
| 3 | 818 | 15.0 | 14.8 | 27.8 | 12.8 | 85.33 | 0.750 | 50.250 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 4 | 1027 | 200.0 | 200.0 | 205.8 | 5.8 | 2.90 | 2.000 | 150.250 | 149.750 | CW |
| 5 | 1636 | 15.0 | 14.0 | 40.7 | 25.7 | 171.33 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 6 | 1845 | 200.0 | 200.0 | 204.8 | 4.8 | 2.40 | 2.000 | 150.250 | 149.750 | CW |
| 7 | 2454 | 15.0 | 14.0 | 40.9 | 25.9 | 172.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 8 | 2663 | 200.0 | 199.9 | 203.0 | 3.0 | 1.50 | 2.000 | 150.250 | 149.750 | CW |
| 9 | 3272 | 15.0 | 14.0 | 41.3 | 26.3 | 175.33 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 10 | 3481 | 200.0 | 200.0 | 204.6 | 4.6 | 2.30 | 2.000 | 150.250 | 149.750 | CW |
| 11 | 4090 | 15.0 | 14.0 | 41.4 | 26.4 | 176.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 12 | 4299 | 200.0 | 200.0 | 201.9 | 1.9 | 0.95 | 2.000 | 150.250 | 149.750 | CW |
| 13 | 4908 | 15.0 | 14.0 | 41.4 | 26.4 | 176.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 14 | 5117 | 200.0 | 200.0 | 202.6 | 2.6 | 1.30 | 2.250 | 150.250 | 149.750 | CW |
| 15 | 5726 | 15.0 | 14.3 | 41.6 | 26.6 | 177.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 16 | 5934 | 200.0 | 200.0 | 202.1 | 2.1 | 1.05 | 2.250 | 150.000 | 150.000 | CW |
| 17 | 6542 | 15.0 | 13.7 | 29.4 | 14.4 | 96.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 18 | 6750 | 200.0 | 200.0 | 202.2 | 2.2 | 1.10 | 2.250 | 150.000 | 150.000 | CW |
| 19 | 7358 | 15.0 | 15.2 | 41.7 | 26.7 | 178.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 20 | 7566 | 200.0 | 200.0 | 202.8 | 2.8 | 1.40 | 2.000 | 150.000 | 150.000 | CW |
| 21 | 8174 | 15.0 | 13.8 | 41.5 | 26.5 | 176.67 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 22 | 8382 | 200.0 | 200.0 | 202.8 | 2.8 | 1.40 | 2.250 | 150.000 | 150.000 | CW |
| 23 | 8990 | 15.0 | 13.6 | 41.6 | 26.6 | 177.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 24 | 9198 | 200.0 | 200.0 | 202.9 | 2.9 | 1.45 | 2.000 | 150.250 | 150.000 | CW |
| 25 | 9807 | 15.0 | 13.9 | 41.7 | 26.7 | 178.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 26 | 10016 | 200.0 | 200.0 | 202.7 | 2.7 | 1.35 | 2.000 | 150.250 | 149.750 | CW |
| 27 | 10625 | 15.0 | 15.1 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 28 | 10834 | 200.0 | 200.0 | 204.5 | 4.5 | 2.25 | 1.250 | 150.250 | 149.750 | CW |
| 29 | 11443 | 15.0 | 13.4 | 41.9 | 26.9 | 179.33 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 30 | 11652 | 200.0 | 200.0 | 203.3 | 3.3 | 1.65 | 2.000 | 150.250 | 149.750 | CW |
| 31 | 12261 | 15.0 | 13.7 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 32 | 12470 | 200.0 | 200.0 | 203.4 | 3.4 | 1.70 | 2.000 | 150.250 | 149.750 | CW |
| 33 | 13079 | 15.0 | 14.1 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 34 | 13288 | 200.0 | 200.0 | 202.8 | 2.8 | 1.40 | 2.000 | 150.250 | 149.750 | CW |
| 35 | 13897 | 15.0 | 13.6 | 41.9 | 26.9 | 179.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 36 | 14105 | 200.0 | 200.0 | 203.1 | 3.1 | 1.55 | 2.000 | 150.000 | 150.000 | CW |
| 37 | 14713 | 15.0 | 13.6 | 41.8 | 26.8 | 178.67 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 38 | 14921 | 200.0 | 200.0 | 201.9 | 1.9 | 0.95 | 1.750 | 150.000 | 150.000 | CW |
| 39 | 15529 | 15.0 | 13.5 | 42.0 | 27.0 | 180.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 40 | 15737 | 200.0 | 200.0 | 204.9 | 4.9 | 2.45 | 2.000 | 150.000 | 150.000 | CW |
| 41 | 16345 | 15.0 | 13.5 | 41.7 | 26.7 | 178.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 42 | 16553 | 200.0 | 200.0 | 204.4 | 4.4 | 2.20 | 1.500 | 150.000 | 150.000 | CW |
| 43 | 17161 | 15.0 | 13.9 | 41.9 | 26.9 | 179.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 44 | 17369 | 200.0 | 200.0 | 204.1 | 4.1 | 2.05 | 2.000 | 150.250 | 150.000 | CW |
| 45 | 17978 | 15.0 | 13.4 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 46 | 18187 | 200.0 | 200.0 | 204.2 | 4.2 | 2.10 | 2.000 | 150.250 | 149.750 | CW |
| 47 | 18796 | 15.0 | 13.7 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 48 | 19005 | 200.0 | 199.9 | 202.8 | 2.8 | 1.40 | 2.250 | 150.250 | 149.750 | CW |
| 49 | 19614 | 15.0 | 14.0 | 35.4 | 20.4 | 136.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 50 | 19823 | 200.0 | 200.0 | 203.9 | 3.9 | 1.95 | 2.000 | 150.250 | 149.750 | CW |
| 51 | 20432 | 15.0 | 13.5 | 41.7 | 26.7 | 178.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 52 | 20641 | 200.0 | 200.0 | 203.9 | 3.9 | 1.95 | 2.000 | 150.250 | 149.750 | CW |
| 53 | 21250 | 15.0 | 14.2 | 31.0 | 16.0 | 106.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 54 | 21459 | 200.0 | 200.0 | 204.6 | 4.6 | 2.30 | 2.000 | 150.250 | 149.750 | CW |
| 55 | 22068 | 15.0 | 14.0 | 41.9 | 26.9 | 179.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 56 | 22276 | 200.0 | 200.0 | 204.3 | 4.3 | 2.15 | 2.000 | 150.000 | 150.000 | CW |
| 57 | 22884 | 15.0 | 13.7 | 41.8 | 26.8 | 178.67 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 58 | 23092 | 200.0 | 200.0 | 204.4 | 4.4 | 2.20 | 2.000 | 150.000 | 150.000 | CW |
| 59 | 23700 | 15.0 | 13.5 | 41.9 | 26.9 | 179.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 60 | 23908 | 200.0 | 200.0 | 203.9 | 3.9 | 1.95 | 2.000 | 150.000 | 150.000 | CW |
| 61 | 24516 | 15.0 | 13.4 | 41.9 | 26.9 | 179.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 62 | 24724 | 200.0 | 199.9 | 202.2 | 2.2 | 1.10 | 2.000 | 150.000 | 150.000 | CW |
| 63 | 25332 | 15.0 | 13.5 | 41.6 | 26.6 | 177.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 64 | 25540 | 200.0 | 199.9 | 202.2 | 2.2 | 1.10 | 2.250 | 150.250 | 150.000 | CW |
| 65 | 26149 | 15.0 | 14.0 | 31.5 | 16.5 | 110.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 66 | 26358 | 200.0 | 200.0 | 202.0 | 2.0 | 1.00 | 2.000 | 150.250 | 149.750 | CW |
| 67 | 26967 | 15.0 | 13.4 | 41.9 | 26.9 | 179.33 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 68 | 27176 | 200.0 | 200.0 | 201.6 | 1.6 | 0.80 | 2.000 | 150.250 | 149.750 | CW |
| 69 | 27785 | 15.0 | 13.6 | 41.9 | 26.9 | 179.33 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 70 | 27994 | 200.0 | 199.9 | 203.4 | 3.4 | 1.70 | 1.750 | 150.250 | 149.750 | CW |
| 71 | 28603 | 15.0 | 13.8 | 29.5 | 14.5 | 96.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 72 | 28812 | 200.0 | 199.9 | 202.4 | 2.4 | 1.20 | 2.000 | 150.250 | 149.750 | CW |
| 73 | 29421 | 15.0 | 13.5 | 41.9 | 26.9 | 179.33 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 74 | 29630 | 200.0 | 200.0 | 204.0 | 4.0 | 2.00 | 2.250 | 150.500 | 149.750 | CW |
| 75 | 30240 | 15.0 | 13.7 | 41.9 | 26.9 | 179.33 | 0.750 | 50.000 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 76 | 30448 | 200.0 | 200.0 | 204.3 | 4.3 | 2.15 | 2.000 | 150.250 | 150.000 | CW |
| 77 | 31057 | 15.0 | 13.8 | 41.8 | 26.8 | 178.67 | 0.750 | 50.000 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 78 | 31265 | 200.0 | 200.0 | 204.0 | 4.0 | 2.00 | 2.000 | 150.250 | 150.000 | CW |
| 79 | 31874 | 15.0 | 13.5 | 42.1 | 27.1 | 180.67 | 0.750 | 50.000 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 80 | 32082 | 200.0 | 200.0 | 203.8 | 3.8 | 1.90 | 1.250 | 150.000 | 150.000 | CW |
| 81 | 32690 | 15.0 | 13.4 | 42.0 | 27.0 | 180.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 82 | 32898 | 200.0 | 200.0 | 203.6 | 3.6 | 1.80 | 2.250 | 150.000 | 150.000 | CW |
| 83 | 33506 | 15.0 | 13.6 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 84 | 33715 | 200.0 | 200.0 | 204.9 | 4.9 | 2.45 | 2.000 | 150.250 | 149.750 | CW |
| 85 | 34324 | 15.0 | 14.2 | 29.8 | 14.8 | 98.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 86 | 34533 | 200.0 | 200.0 | 204.3 | 4.3 | 2.15 | 2.000 | 150.250 | 149.750 | CW |
| 87 | 35142 | 15.0 | 13.9 | 41.7 | 26.7 | 178.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 88 | 35351 | 200.0 | 200.0 | 203.6 | 3.6 | 1.80 | 2.000 | 150.250 | 149.750 | CW |
| 89 | 35960 | 15.0 | 13.8 | 30.6 | 15.6 | 104.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 90 | 36169 | 200.0 | 200.0 | 204.2 | 4.2 | 2.10 | 2.250 | 150.250 | 149.750 | CW |
| 91 | 36778 | 15.0 | 13.5 | 41.7 | 26.7 | 178.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 92 | 36987 | 200.0 | 200.0 | 208.3 | 8.3 | 4.15 | 1.250 | 150.250 | 149.750 | CW |
| 93 | 37596 | 15.0 | 14.3 | 32.7 | 17.7 | 118.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 94 | 37804 | 200.0 | 200.0 | 204.2 | 4.2 | 2.10 | 2.000 | 150.000 | 150.000 | CW |
| 95 | 38412 | 15.0 | 13.4 | 29.5 | 14.5 | 96.67 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 96 | 38620 | 200.0 | 200.0 | 204.3 | 4.3 | 2.15 | 2.000 | 150.000 | 150.000 | CW |
| 97 | 39228 | 15.0 | 13.7 | 41.7 | 26.7 | 178.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 98 | 39436 | 200.0 | 200.0 | 203.4 | 3.4 | 1.70 | 2.250 | 150.000 | 150.000 | CW |
| 99 | 40044 | 15.0 | 13.5 | 41.7 | 26.7 | 178.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 100 | 40252 | 200.0 | 200.0 | 204.3 | 4.3 | 2.15 | 2.250 | 150.000 | 150.000 | CW |
| 101 | 40860 | 15.0 | 13.4 | 41.7 | 26.7 | 178.00 | 0.750 | 50.250 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 102 | 41069 | 200.0 | 200.0 | 203.7 | 3.7 | 1.85 | 1.250 | 150.250 | 149.750 | CW |
| 103 | 41678 | 15.0 | 13.4 | 41.7 | 26.7 | 178.00 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 104 | 41887 | 200.0 | 199.9 | 203.5 | 3.5 | 1.75 | 1.500 | 150.250 | 149.750 | CW |
| 105 | 42496 | 15.0 | 14.7 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 106 | 42705 | 200.0 | 200.0 | 204.0 | 4.0 | 2.00 | 2.750 | 150.250 | 149.750 | CW |
| 107 | 43314 | 15.0 | 13.7 | 41.8 | 26.8 | 178.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 108 | 43523 | 200.0 | 200.0 | 205.0 | 5.0 | 2.50 | 2.000 | 150.500 | 149.750 | CW |
| 109 | 44133 | 15.0 | 13.9 | 30.2 | 15.2 | 101.33 | 0.750 | 50.250 | 349.500 | Pulse, F=0.400kHz, D=15.0% |
| 110 | 44342 | 200.0 | 200.0 | 204.5 | 4.5 | 2.25 | 1.750 | 150.250 | 149.750 | CW |
| 111 | 44951 | 15.0 | 14.2 | 29.5 | 14.5 | 96.67 | 0.750 | 50.250 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 112 | 45160 | 200.0 | 199.9 | 203.0 | 3.0 | 1.50 | 2.000 | 150.500 | 149.750 | CW |
| 113 | 45770 | 15.0 | 13.9 | 30.0 | 15.0 | 100.00 | 0.750 | 50.000 | 349.750 | Pulse, F=0.400kHz, D=15.0% |
| 114 | 45978 | 200.0 | 200.0 | 203.9 | 3.9 | 1.95 | 1.750 | 150.000 | 150.000 | CW |
| 115 | 46586 | 15.0 | 13.5 | 30.8 | 15.8 | 105.33 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 116 | 46794 | 200.0 | 200.0 | 204.2 | 4.2 | 2.10 | 1.250 | 150.000 | 150.000 | CW |
| 117 | 47402 | 15.0 | 13.6 | 41.7 | 26.7 | 178.00 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 118 | 47610 | 200.0 | 200.0 | 203.0 | 3.0 | 1.50 | 2.000 | 150.000 | 150.000 | CW |
| 119 | 48218 | 15.0 | 13.5 | 30.4 | 15.4 | 102.67 | 0.750 | 50.000 | 350.000 | Pulse, F=0.400kHz, D=15.0% |
| 120 | 48426 | 200.0 | 200.0 | 204.1 | 4.1 | 2.05 | 2.000 | 150.000 | 150.000 | CW |

Detected RF cycles: 120. The plot shows user assigned time range.

# Step 4 — Forward Power vs Setpoint (Tykon only)

Quantum: no setpoint, skip Step 4

# Step 5 — Reflected Power (Pref high)

Rule: Pref > 1.00% of Pfwd after masking the first 3.0 ms.

Flagged cycles: 60

| Cycle | Start row() | First bad row() | Worst row() | Worst Pref (W) | Worst Pref/Pfwd (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 25 | 175 | 11.10 | 68.10 |
| 3 | 819 | 831 | 1011 | 10.70 | 68.15 |
| 5 | 1637 | 1648 | 1699 | 13.00 | 70.27 |
| 7 | 2455 | 2467 | 2507 | 13.20 | 69.47 |
| 9 | 3273 | 3284 | 3345 | 12.70 | 69.78 |
| 11 | 4091 | 4103 | 4143 | 13.30 | 70.00 |
| 13 | 4909 | 4921 | 4961 | 13.30 | 70.00 |
| 15 | 5727 | 5739 | 5799 | 12.70 | 69.78 |
| 17 | 6543 | 6558 | 6685 | 12.10 | 69.54 |
| 19 | 7359 | 7371 | 7411 | 12.90 | 69.73 |
| 21 | 8175 | 8187 | 8237 | 13.20 | 70.59 |
| 23 | 8991 | 9003 | 9033 | 14.10 | 71.21 |
| 25 | 9808 | 9820 | 9860 | 13.40 | 70.53 |
| 27 | 10626 | 10638 | 10678 | 12.90 | 69.73 |
| 29 | 11444 | 11456 | 11496 | 13.90 | 71.28 |
| 31 | 12262 | 12274 | 12304 | 14.00 | 71.07 |
| 33 | 13080 | 13092 | 13152 | 12.70 | 69.78 |
| 35 | 13898 | 13910 | 13940 | 14.10 | 71.57 |
| 37 | 14714 | 14726 | 14756 | 14.00 | 71.07 |
| 39 | 15530 | 15542 | 15582 | 13.70 | 70.98 |
| 41 | 16346 | 16358 | 16388 | 14.30 | 71.50 |
| 43 | 17162 | 17174 | 17224 | 13.20 | 70.21 |
| 45 | 17979 | 17991 | 18021 | 14.30 | 71.50 |
| 47 | 18797 | 18809 | 18839 | 14.00 | 71.07 |
| 49 | 19615 | 19627 | 19677 | 13.10 | 70.43 |
| 51 | 20433 | 20445 | 20475 | 14.20 | 71.36 |
| 53 | 21251 | 21263 | 21343 | 12.40 | 69.66 |
| 55 | 22069 | 22081 | 22141 | 13.00 | 70.27 |
| 57 | 22885 | 22897 | 22927 | 13.90 | 70.92 |
| 59 | 23701 | 23713 | 23753 | 13.80 | 71.13 |
| 61 | 24517 | 24529 | 24559 | 13.80 | 70.77 |
| 63 | 25333 | 25345 | 25375 | 14.10 | 71.21 |
| 65 | 26150 | 26162 | 26202 | 13.20 | 70.21 |
| 67 | 26968 | 26980 | 27010 | 14.00 | 71.43 |
| 69 | 27786 | 27798 | 27828 | 14.20 | 71.36 |
| 71 | 28604 | 28620 | 28676 | 12.80 | 69.95 |
| 73 | 29422 | 29434 | 29474 | 13.90 | 71.28 |
| 75 | 30241 | 30253 | 30273 | 14.30 | 70.79 |
| 77 | 31058 | 31070 | 31090 | 14.50 | 70.73 |
| 79 | 31875 | 31887 | 31917 | 14.30 | 71.50 |
| 81 | 32691 | 32703 | 32733 | 13.90 | 71.28 |
| 83 | 33507 | 33519 | 33539 | 14.50 | 71.43 |
| 85 | 34325 | 34337 | 34397 | 12.90 | 70.49 |
| 87 | 35143 | 35155 | 35195 | 13.10 | 70.43 |
| 89 | 35961 | 35973 | 36033 | 13.30 | 70.74 |
| 91 | 36779 | 36791 | 36821 | 14.20 | 71.36 |
| 93 | 37597 | 37609 | 37649 | 13.30 | 70.74 |
| 95 | 38413 | 38425 | 38465 | 13.80 | 71.13 |
| 97 | 39229 | 39241 | 39291 | 13.40 | 70.90 |
| 99 | 40045 | 40057 | 40097 | 13.60 | 70.83 |
| 101 | 40861 | 40873 | 40913 | 14.20 | 71.72 |
| 103 | 41679 | 41691 | 41711 | 14.60 | 71.92 |
| 105 | 42497 | 42509 | 42539 | 13.00 | 70.27 |
| 107 | 43315 | 43327 | 43347 | 14.30 | 71.50 |
| 109 | 44134 | 44146 | 44216 | 13.10 | 70.43 |
| 111 | 44952 | 44964 | 45024 | 13.00 | 70.27 |
| 113 | 45771 | 45783 | 45823 | 13.50 | 70.68 |
| 115 | 46587 | 46599 | 46629 | 14.20 | 71.72 |
| 117 | 47403 | 47415 | 47455 | 13.60 | 70.83 |
| 119 | 48219 | 48232 | 48281 | 13.60 | 70.83 |

Note: table capped at 200 rows.

# Pulse Mode Range Alarms

Pulse-mode rows: 10712

Alarms: 0